Particle Swarm Optimization for 3D Node Localization in Wireless Sensor Networks

Review of 10 research papers (2008-2025) supporting the p09_pso_localization_3d.py implementation

# 1. Introduction

Our project code (p09_pso_localization_3d.py) localizes an unknown sensor node in a 60 x 60 x 20 m field from noisy range measurements to four anchors. A swarm of 20 particles is flown for 60 iterations by a simple velocity rule (0.7 inertia plus a random 1.4-weighted pull towards the current best particle), and each particle is scored by the sum of squared residuals between its anchor distances and the measured distances; the whole search runs centrally at the base station and returns a 3D estimate within about 0.9 m of the true position. The ten papers reviewed below trace exactly this idea from its origin (Gopakumar & Jacob, 2008) through parameter studies, anchor-selection and niching refinements, to the latest 3D, hybrid and adaptive PSO localizers of 2023-2025. For each paper the table records the objective, two advantages and two disadvantages; the recurring weaknesses are then consolidated and mapped to the concrete improvements our project will implement.

# 2. Paper Review Table

| S.No. | Author(s) & Year | Objective | Advantages (2 lines) | Disadvantages (2 lines) |
| --- | --- | --- | --- | --- |
| 1 | Gopakumar & Jacob (2008) IET Int. Conf. Wireless, Mobile & Multimedia Networks | Formulate WSN node localization as a non-linear least-squares problem and solve it centrally with PSO ("PSO-Loc") from noisy beacon range estimates; benchmarked against simulated annealing. | 1. Simple, derivative-free squared-residual fitness - the same cost used in our code. 2. Localization error more than halved, with faster convergence, versus simulated annealing. | 1. Centralized and 2D; PSO dimension grows 2x with target count, hurting scalability. 2. Ignores flip ambiguity and nodes with < 3 beacons; simulation only. |
| 2 | Kulkarni & Venayagamoorthy (2011) IEEE Trans. SMC-Part C 41(2) | Survey PSO variants and their use for WSN deployment, localization, clustering and data aggregation; contrasts PSO-Loc, PSO-Iterative, beaconless and 4-beacon localizers. | 1. Positions PSO as a light, easy-to-code optimizer for resource-limited nodes. 2. Flags open issues - flip ambiguity, error accumulation, distributed PSO - still relevant today. | 1. Survey only: no new algorithm and no 3D results. 2. Notes that iterative PSO cost and centralization limit scalability and real-time use. |
| 3 | Cui, Shu, Song & Wang (2017) Sensors 17(3):487 | Systematically select PSO parameters (inertia, acceleration coefficients, swarm size, topology) and compare nine PSO variants for range-based localization against SOCP. | 1. Evidence-based parameter choices; constriction PSO with ring topology performed best. 2. Outperformed SOCP in accuracy for iterative multi-hop localization. | 1. 2D static networks only; range noise assumed Gaussian. 2. Iterative localization accumulates error; simulation-based validation. |
| 4 | Zhou, Cui, Cao & Liang (2018) Wireless Commun. Mobile Comput. 2018:2473875 | Localize large-scale WSNs with unevenly deployed anchors using niching PSO plus three-criteria (distance, angle, precision) selection of a minimal set of reliable anchors. | 1. Niching resolves flip ambiguity caused by near-collinear anchors. 2. Fewer references cut computation; higher accuracy and lower time than other PSO localizers. | 1. 2D and simulation only; extra selection logic and message exchange. 2. Relies on precision estimates of already-localized nodes, so errors can propagate. |
| 5 | Jia, Yang & Zhang (2022) Recent Adv. Comput. Sci. Commun. 15(9) | Improve 3D DV-Hop localization by refining hop distance / communication radius and solving coordinates with an improved PSO (mutation factor, adaptive learning factor and inertia weight). | 1. Range-free 3D scheme - no ranging hardware required. 2. Better convergence, accuracy and robustness than comparable 3D DV-Hop variants. | 1. Authors acknowledge a higher computational load. 2. Hop-based distances stay coarse in anisotropic fields; simulation only, closed access. |
| 6 | Lee, Cheng, Lin & Huang (2023) Electronics 12(4):905 | Localize and track a moving target with PSO on an RSSI path-loss model, adding Region Segmentation (RSM) and Dynamic Individual Selection (DIS) to cut particles and run time. | 1. RSM cut localization time 48.95% and tracking time 34.14% with no accuracy loss. 2. Adaptive inertia plus DIS reached 96.8% tracking success with few particles. | 1. 2D 100 x 100 m indoor case with 4 anchors; idealized RSSI channel model. 2. Simulation only; no hardware or 3D validation. |
| 7 | Alhasan, Ahmad, Wazirali, Aleisa & Abo Shdeed (2023) J. King Saud Univ. - Comput. Inf. Sci. 35(9):101782 | Propose AMCMPSO, a centre-of-mass-inspired PSO with adaptive inertia and acceleration coefficients for range-based auto-localization on regular and irregular 3D surfaces (6LoWPAN / Zigbee). | 1. Fully 3D and range-based; adaptive coefficients speed convergence and avoid stagnation. 2. Reports error below 1.34 cm and > 87% coverage in 3D scenarios. | 1. Simulation only; very high accuracy claims are not validated on hardware. 2. Extra centre-of-mass computation and per-iteration parameter adaptation. |
| 8 | Kaur, Prashar, Khan, Kadry & Kim (2025) PeerJ Computer Science 11:e2770 | PSLDV-Hop: combine DV-Hop distance estimates with PSO and an iterative refinement stage to lower localization error under noise, NLOS and varied anchor layouts. | 1. Consistently lower error than DV-Hop, GA-DV-Hop and PSO-DV-Hop for ranges of 20-40 units. 2. Robust to node count, anchor placement, measurement noise and obstacles. | 1. Heavy: up to 100 particles, 50 PSO and 100 refinement iterations per node. 2. 2D MATLAB simulation only; range-free base limits attainable accuracy. |
| 9 | Singh, Yadav, Rishiwal, Gautam, Shankhdhar & Kumar (2025) Int. J. Math. Eng. Manag. Sci. 10(6) | Hybrid 3D localization for WSN-assisted IoT in smart agriculture: Random-Forest RSSI anomaly filtering, PSO position search and trilateration refinement. | 1. 3D, range-based; reports about 1.2 m error, 8-10 J energy and < 0.5 s computation. 2. Anomaly filtering makes PSO robust to noisy RSSI in large deployments. | 1. Synthetic data only; assumes static nodes, known path-loss exponent and perfect sync. 2. Random-Forest stage adds training cost; no field trials. |
| 10 | Zhang, Dong & Wang (2025) arXiv:2507.22317 | AdapSCA-PSO: adaptively switch between Sine-Cosine (global) and PSO (local) search with hop-aware initialization and a redesigned fitness for IoT WSN node localization. | 1. 84.97% lower average error than plain PSO / SCA-PSO with far fewer iterations. 2. Hop-based initialization near the nearest anchor stabilizes the search; suits distributed use. | 1. Preprint; simulation only in a 2D 100 x 100 m field with hop-based ranging. 2. Switching logic adds parameters and per-node overhead. |

PDFs / metadata stubs for all ten papers are provided in PSO_3D_Localization_WSN_Papers.zip; full citations with DOIs appear in Section 5.

# 3. Common Disadvantages Across Papers

Reading the ten studies side by side, six limitations recur regardless of publication year or PSO variant:

- Computational and energy cost. Every paper runs tens of particles for tens to hundreds of iterations per node (e.g. 100 particles x 50 + 100 refinement iterations in PSLDV-Hop); on battery-powered motes this drains energy and delays position fixes.

- Premature convergence to local optima. Basic PSO with fixed inertia and a single global attractor stagnates; most papers (Cui 2017, Jia 2022, Alhasan 2023, Zhang 2025) bolt on adaptive weights, mutation or hybrid search to escape it.

- Sensitivity to anchor geometry and ranging noise. Near-collinear or coplanar anchors create flip / mirror ambiguity (Gopakumar 2008, Zhou 2018) and noisy RSSI or hop-count distances translate directly into position error.

- Poor scalability and centralized operation. Solving at a base station requires every range to be forwarded (Gopakumar 2008, Kulkarni 2011) and search dimension grows with the number of targets; iterative schemes accumulate error.

- Simulation-only, mostly 2D validation. Seven of the ten studies work in 2D (only Jia 2022, Alhasan 2023 and Singh 2025 are 3D); all rely on MATLAB/Python simulations with idealized channel models, static nodes and perfect synchronization; none reports hardware field trials.

- Weak, ad-hoc parameter tuning. Inertia, acceleration coefficients, swarm size and stop criteria are hand-picked and rarely justified for 3D fields, so results transfer poorly between deployments.

# 4. Our Project Focus: Fixing These Gaps

Each recurring disadvantage above maps to a concrete change we will implement and evaluate in p09_pso_localization_3d.py, so that every gap can be traced back to a project contribution:

- Cheaper, energy-aware search. Keep the swarm small (20 particles) but add early stopping on fitness plateau, warm-start particles from the trilateration/least-squares seed, and profile iterations x particles as an energy proxy so the 60-iteration budget is justified, not assumed.

- Escape local optima. Replace the fixed 0.7 inertia / 1.4 attraction with linearly decreasing inertia, add personal-best (cognitive) memory alongside the global best, and inject a small mutation / re-initialisation for stagnant particles, following Cui (2017) and Alhasan (2023).

- Robustness to anchor geometry and noise. Use a weighted residual fitness (weights inversely proportional to range variance), check the anchor Geometric Dilution of Precision (GDOP) before solving, and run multi-start niching (Zhou 2018) to detect mirror solutions in the 3D field.

- Scalability and distributed operation. Solve each node independently with only its neighbouring anchors (3-D search per node instead of one 3N-D search), then let confidently localized nodes act as secondary anchors with precision-based selection to limit error propagation.

- Realistic 3D validation. Move beyond a single true position: Monte-Carlo runs over random node positions, Gaussian and NLOS-biased range errors, obstacles and anchor drop-outs in the 60 x 60 x 20 m field, reporting RMSE, CDF of error and convergence curves; then validate on a small real testbed (e.g. UWB/BLE ranging) as the papers do not.

- Principled parameter selection. Grid-search inertia, acceleration coefficients and swarm size on the 3D scenario, report sensitivity plots, and adopt the constriction-coefficient / ring-topology settings recommended by Cui (2017) as the baseline instead of hand-picked constants.

Together these changes turn the baseline demonstrator into an energy-aware, geometry-checked, distributed 3D PSO localizer whose parameters and robustness are justified by experiment rather than assumption - directly addressing the gaps left open by the reviewed literature.

# 5. References

[1] Gopakumar, A., & Jacob, L. (2008). Localization in wireless sensor networks using particle swarm optimization. IET International Conference on Wireless, Mobile and Multimedia Networks (WMMN 2008), Mumbai, India, pp. 227-230. https://doi.org/10.1049/cp:20080185

[2] Kulkarni, R. V., & Venayagamoorthy, G. K. (2011). Particle swarm optimization in wireless-sensor networks: A brief survey. IEEE Transactions on Systems, Man, and Cybernetics, Part C, 41(2), 262-267. https://doi.org/10.1109/TSMCC.2010.2054080

[3] Cui, H., Shu, M., Song, M., & Wang, Y. (2017). Parameter selection and performance comparison of particle swarm optimization in sensor networks localization. Sensors, 17(3), 487. https://doi.org/10.3390/s17030487

[4] Zhou, C., Cui, H., Cao, N., & Liang, Y. (2018). Localization of large-scale wireless sensor networks using niching particle swarm optimization and reliable anchor selection. Wireless Communications and Mobile Computing, 2018, 2473875. https://doi.org/10.1155/2018/2473875

[5] Jia, S., Yang, C., & Zhang, H. (2022). Three-dimensional node localization algorithm based on communication radius refinement and improved particle swarm optimization. Recent Advances in Computer Science and Communications, 15(9), 1245-1258. https://doi.org/10.2174/2666255814666210617142339

[6] Lee, S.-H., Cheng, C.-H., Lin, C.-C., & Huang, Y.-F. (2023). PSO-based target localization and tracking in wireless sensor networks. Electronics, 12(4), 905. https://doi.org/10.3390/electronics12040905

[7] Alhasan, W., Ahmad, R., Wazirali, R., Aleisa, N., & Abo Shdeed, W. (2023). Adaptive mean center of mass particle swarm optimizer for auto-localization in 3D wireless sensor networks. Journal of King Saud University - Computer and Information Sciences, 35(9), 101782. https://doi.org/10.1016/j.jksuci.2023.101782

[8] Kaur, B., Prashar, D., Khan, A. A., Kadry, S., & Kim, J. (2025). PSLDV-Hop: a robust localization algorithm for WSN using PSO and refinement process. PeerJ Computer Science, 11, e2770. https://doi.org/10.7717/peerj-cs.2770

[9] Singh, H., Yadav, P., Rishiwal, V., Gautam, A., Shankhdhar, A., & Kumar, K. (2025). Hybrid optimized 3D localization for WSN-assisted IoT networks in smart agriculture. International Journal of Mathematical, Engineering and Management Sciences, 10(6), 1967-1988. https://doi.org/10.33889/IJMEMS.2025.10.6.091

[10] Zhang, Z., Dong, Q., & Wang, W. (2025). AdapSCA-PSO: An adaptive localization algorithm with AI-based hybrid SCA-PSO for IoT WSNs. arXiv preprint arXiv:2507.22317. https://doi.org/10.48550/arXiv.2507.22317